- Documentation coscomekg.dll version 1.0.0

Documentation coscomekg.dll version 1.0.0	1

Function overview	1

coscomopenPort	1

coscomclosePort	1

coscomgetVersion	2

coscomgetStatus	2

coscomgetError	2

coscomgetDuration	2

coscomgetSpeed	2

coscomgetAcceleration	2

coscomgetElevation	3

coscomgetHeartrate	3

coscomgetDistance	3

coscomgetFailSafe	3

coscomsetFailSafe	3

coscomsetSpeed	3

coscomsetAcceleration	3

coscomsetElevation	3

coscomsetSpeedAccel	4

coscomKeyPressStart	4

coscomKeyPressStop	4

coscomKeyPressPlus	4

coscomKeyPressMinus	4

coscomKeyPressUp	4

coscomKeyPressDown	4

coscomsetProtocol	4

coscomgetProtocol	5

How to use coscomekg.dll	5

1. Delphi	5

2. C/ C++	6

# Function overview

The return value of all functions is a error indicator described as follows:

1: Successful operation

0: No Connection was assigned to device

-1: Unsupported function at this device

-2: Connection is interrupted

-3: Exception

-4: Wrong port number was used

### coscomopenPort

int coscomopenPort(int portnumber);

function coscomopenPort(portnumber : Integer) : Integer;

connects device with COM Port 1..n

### coscomclosePort

int coscomclosePort(int portnumber);

function coscomclosePort(portnumber : Integer) : Integer;

closes connection at COM Port 1..n

### coscomgetVersion

int coscomgetVersion(int portnumber;char * versionchars);

function coscomgetVersion(portnumber : Integer;versionchars : PCHAR) : Integer;

versionchars: 3 x Bytes (ASCII Chars)

Release.Version * 100

### coscomgetStatus

int coscomgetStatus(int portnumber;int *value);

function coscomgetStatus(portnumber : Integer;value : PInteger) : Integer;

value:

0 Stop

1 Run

2 Pause

### coscomgetError

int coscomgetError(int portnumber;int *value);

function coscomgetError(portnumber : Integer;value : PInteger) : Integer;

value = Error Number:

1 Oil Interval Exceeded

2 Service Interval Exceeded

21 Elevator Error (Treadmill)

30 Drive Error (Treadmill)

50 Drive Error (Treadmill)

### coscomgetDuration

int coscomgetDuration(int portnumber;int *value);

function coscomgetDuration(portnumber : Integer;value : PInteger) : Integer;

value: duration in seconds (since last start command)

### coscomgetSpeed

int coscomgetSpeed(int portnumber;double *value);

function coscomgetSpeed(portnumber : Integer;value : PDouble) : Integer;

value: actual speed in meter per seconds

### coscomgetAcceleration

int coscomgetAcceleration(int portnumber;int *value);

function coscomgetAcceleration(portnumber : Integer;value : PInteger) : Integer;

value: actual accelerationindex

0 None

1 = 131 s

2 = 66 s

3 = 33 s

4 = 16 s

5 = 8 s

6 = 5 s

7 = 3 s

The selected time relates to the duration of a speed change from standstill to maximum speed or vice versa. The exact value is calculated as follows:
a = v / t = maximum speed / acceleration time. The new acceleration value is used with the next "S02" command.

Example:
Acceleration index: 3 = 33 s
Maximum treadmill speed: 22 km/h = 6.11 m/s
Acceleration value: a = (6.11 / 33) m/s² = 0.185 m/s²

### coscomgetElevation

int coscomgetElevation(int portnumber;double *value);

function coscomgetElevation(portnumber : Integer;value : PDouble) : Integer;

value: actual elevation in  %

### coscomgetHeartrate

int coscomgetHeartrate(int portnumber;int *value);

function coscomgetHeartrate(portnumber : Integer;value : PInteger) : Integer;

value: actual heart frequency in beats per minute

### coscomgetDistance

int coscomgetDistance(int portnumber;int *value);

function coscomgetDistance(portnumber : Integer;value : PInteger) : Integer;

value: actual distance in meter ( since last start command )

### coscomgetFailSafe

int coscomgetFailSafe(int portnumber;int *value);

function coscomgetFailSafe(portnumber : Integer;value : PInteger) : Integer;

Reads communication timeout for failsafe (STOP)

value:

0: Disable Failsafe (Default)
1 - 250 1/10 seconds (= 0.1 - 25.0 seconds):

### coscomsetFailSafe

int coscomsetFailSafe(int portnumber;int value);

function coscomsetFailSafe(portnumber : Integer;value : Integer) : Integer;

Enables communication timeout for failsafe (STOP)

value:

0: Disable Failsafe (Default)
1 - 250 1/10 seconds (= 0.1 - 25.0 seconds):

### coscomsetSpeed

int coscomsetSpeed(int portnumber;value : Double);

function coscomsetSpeed(portnumber : Integer;value : Double) : Integer;

value:  speed in meter per seconds

### coscomsetAcceleration

int coscomsetAcceleration(int portnumber;int value);

function coscomsetAcceleration(portnumber : Integer;value : Integer) : Integer;

value: acceleration in acceleration indices ( see coscomgetAcceleration)

Only activated with new coscomsetSpeed function!

### coscomsetElevation

int coscomsetElevation(int portnumber;double value);

function coscomsetElevation(portnumber : Integer;value : Double) : Integer;

value: elevation in %

### coscomsetSpeedAccel

int coscomsetSpeedAccel(int portnumber;double value;int accelerationindex);

function coscomsetSpeedAccel(portnumber : Integer;value : Double;accelerationindex : Integer) : Integer;

values: speed in meter per seconds

acceleration in acceleration indices ( see coscomgetAcceleration)

### coscomKeyPressStart

int coscomKeyPressStart(int portnumber;int how);

function coscomKeyPressStart(portnumber : Integer;how : Integer) : Integer;

how:

1: key pressed

0: key released

### coscomKeyPressStop

int coscomKeyPressStop(int portnumber;int how);

function coscomKeyPressStop(portnumber : Integer;how : Integer) : Integer;

how:

1: key pressed

0: key released

### coscomKeyPressPlus

int coscomKeyPressPlus(int portnumber;int how);

function coscomKeyPressPlus(portnumber : Integer;how : Integer) : Integer;

how:

1: key pressed

0: key released

### coscomKeyPressMinus

int coscomKeyPressMinus(int portnumber;int how);

function coscomKeyPressMinus(portnumber : Integer;how : Integer) : Integer;

how:

1: key pressed

0: key released

### coscomKeyPressUp

int coscomKeyPressUp(int portnumber;int how);

function coscomKeyPressUp(portnumber : Integer;how : Integer) : Integer;

how:

1: key pressed

0: key released

### coscomKeyPressDown

int coscomKeyPressDown(int portnumber;int how);

function coscomKeyPressDown(portnumber : Integer;how : Integer) : Integer;

how:

1: key pressed

0: key released

### coscomsetProtocol

int coscomsetProtocol(int portnumber;int how);

function coscomsetProtocol(portnumber : Integer;how : Integer) : Integer;

Activates communication protocol.

how:

1 On

2 Off

### coscomgetProtocol

int coscomgetProtocol(int portnumber;int *how);

function coscomgetProtocol(portnumber : Integer;how : PInteger) : Integer;

Activates communication protocol.

how:

1 On

2 Off

### How to use coscomekg.dll

## 1. Delphi

- Make sure that the coscomekg.dll is in your application path.

- Add coscomdll.pas to your project.

- Sample code open communication:

procedure TTestcoscomdllForm.ActionOpenDeviceExecute(Sender: TObject);

var

aportnumber : Integer;

begin

closeDevice;

try

aportnumber := StrToInt(Edit1.Text);

except

exit;

end;

if coscomopenPort(aportnumber) > 0 then

FPortNumber := aportnumber;

end;

- Sample code close communication:

procedure TTestcoscomdllForm.closeDevice;

begin

if (FPortNumber > 0) and (coscomclosePort(FPortNumber) > 0) then

FPortNumber := 0;

end;

- Sample code read status

procedure TTestcoscomdllForm.ActionGetStatusExecute(Sender: TObject);

var

myvalue,arowindex : Integer;

begin

arowindex := 1;

FLastError := coscomgetStatus(FPortNumber,@myvalue);

if FLastError > 0 then

begin

with StringGrid1 do

begin

Cells[0,FixedRows + arowindex] := 'Status';

Cells[1,FixedRows + arowindex] := Format('%d',[myvalue]);

end;

end

else

begin

with StringGrid1 do

begin

Cells[0,FixedRows + arowindex] := 'Fail Safe';

case FLastError of

0: Cells[1,FixedRows + arowindex] := 'Connection Off';

-1: Cells[1,FixedRows + arowindex] := 'Function unsupported';

-2: Cells[1,FixedRows + arowindex] := 'Connection interrupted';

-3: Cells[1,FixedRows + arowindex] := 'Exception';

-4: Cells[1,FixedRows + arowindex] := 'wrong port number';

end;

end;

end;

end;

- Sample code set speed

procedure TTestcoscomdllForm.ActionGetSpeedExecute(Sender: TObject);

var

myvalue : Double;

begin

if coscomgetSpeed(FPortNumber,@myvalue) > 0 then

begin

with StringGrid1 do

begin

Cells[0,FixedRows + 2] := 'Speed';

Cells[1,FixedRows + 2] := FormatFloat('0.0',myvalue);

end;

end

else

begin

with StringGrid1 do

begin

Cells[0,FixedRows + 2] := 'Speed';

Cells[1,FixedRows + 2] := 'Error';

end;

end;

end;

- Sample code key functions

procedure TTestcoscomdllForm.Button1MouseDown(Sender: TObject;

Button: TMouseButton; Shift: TShiftState; X, Y: Integer);

begin

coscomKeyPressPlus(FPortNumber,1);

end;

procedure TTestcoscomdllForm.Button1MouseUp(Sender: TObject;

Button: TMouseButton; Shift: TShiftState; X, Y: Integer);

begin

coscomKeyPressPlus(FPortNumber,0);

end;

## 2. C/ C++

- Make sure that the coscomekg.dll is in your application path.

- Add coscomdll.cpp to your project.

- Use multithread options to compile your project!

- Make sure  loadcoscomEKGDLL() and unloadcoscomEKGDLL() is invoked only once. Do not use the instance initialization or clean up functions of the application!
Sample C++ Builder:

WINAPI WinMain(HINSTANCE, HINSTANCE, LPSTR, int)

{

try

{

Application->Initialize();

loadcoscomEKGDLL();

Application->CreateForm(__classid(Tformborlandtestegk), &formborlandtestegk);

Application->Run();

}

catch (Exception &exception)

{

Application->ShowException(&exception);

}

unloadcoscomEKGDLL();

return 0;

}

Sample Visual C++:

BOOL CMfctestcoscomdllApp::InitApplication()

{

// TODO: Speziellen Code hier einfügen und/oder Basisklasse aufrufen

loadcoscomEKGDLL();

return CWinApp::InitApplication();

}

void CMfctestcoscomdllApp::OnFinalRelease()

{

// TODO: Speziellen Code hier einfügen und/oder Basisklasse aufrufen

unloadcoscomEKGDLL();

CWinApp::OnFinalRelease();

}

- Sample code open communication:

void __fastcall Tformborlandtestegk::OpenDevice1Click(TObject *Sender)

{

char StrComPort[20];

int Size = Edit1->GetTextLen();

Edit1->GetTextBuf(StrComPort,Size+1);

int aportnumber = atoi(StrComPort);

closeDevice();

if (coscomopenPort(aportnumber) > 0 )

{

FPortNumber = aportnumber;

}

}

- Sample code close communication:

void Tformborlandtestegk::closeDevice()

{

if ((FPortNumber > 0) && (coscomclosePort(FPortNumber) > 0))

FPortNumber = 0;

}

- Sample code read status

void __fastcall Tformborlandtestegk::ReadStatus1Click(TObject *Sender)

{

int myValue;

int arowindex = 1;

StringGrid1->Cells[0][StringGrid1->FixedRows + arowindex] = "Status";

FLastError = coscomgetStatus(FPortNumber,&myValue);

if (FLastError > 0)

{

StringGrid1->Cells[1][StringGrid1->FixedRows + arowindex] = FormatFloat("0",myValue);

}

else

{

switch (FLastError) {

case 0: StringGrid1->Cells[1][StringGrid1->FixedRows + arowindex] = "Connection Off"; break;

case -1: StringGrid1->Cells[1][StringGrid1->FixedRows + arowindex] = "Function unsupported"; break;

case -2: StringGrid1->Cells[1][StringGrid1->FixedRows + arowindex] = "Connection interrupted"; break;

case -3: StringGrid1->Cells[1][StringGrid1->FixedRows + arowindex] = "Exception"; break;

case -4: StringGrid1->Cells[1][StringGrid1->FixedRows + arowindex] = "wrong Portnumber"; break;

}

}

}

- Sample code set speed

void __fastcall Tformborlandtestegk::SetSpeed1Click(TObject *Sender)

{

double myValue;

try

{

myValue = StrToFloat(StringGrid2->Cells[1][StringGrid2->FixedRows]);

if (coscomsetSpeed(FPortNumber,myValue) > 0)

StringGrid2->Cells[0][StringGrid2->FixedRows] = "Speed Ok";

else

StringGrid2->Cells[0][StringGrid2->FixedRows] = "Speed failed";

}

catch(...)

{

StringGrid2->Cells[0][StringGrid2->FixedRows] = "Speed failed";

}

}

- Sample code key functions

void __fastcall Tformborlandtestegk::Button1MouseDown(TObject *Sender,

TMouseButton Button, TShiftState Shift, int X, int Y)

{

coscomKeyPressMinus(FPortNumber,1);

}

//---------------------------------------------------------------------------

void __fastcall Tformborlandtestegk::Button1MouseUp(TObject *Sender,

TMouseButton Button, TShiftState Shift, int X, int Y)

{

coscomKeyPressMinus(FPortNumber,0);

}